CNC Foam cutter FR-DP

Functional requirements:

- Create predefined wing profiles with precision +/- 0,5mm.

- Machine has to be stiff enough to be able to create exactly the same profile repeatedly, with in tolerances about +/- 0,5mm.

- The ratio between heat of the wire and movement has to be right.

- The tension on the wire has to be constant with tolerances about +/- 10N.

- The machine has to have four degrees of freedom.

Design Parameters Hardware:

1.	Use hot resistance wire to cut through foam pieces.

5.	Each end of wire can move independently in two parallel planes.

4.	Wire has a spring mechanism to be able to elongate / shorten without losing tension.

5.	Four stepper motors will be used to create x-y travel in each plane.

1.2.	Travel center of action must be designed in a way that prevents jamming and bending of travel mechanism to prevent inaccurate results.

Design Parameters Software:

1.	Define profiles for each end of cutter, and define x-y coordinates of one point of each profile (reference point).

1.	Ensure movement of wire starts at ref point for each profile.

1.	Speed of movement must be proportionally the same on each end for correct lofting. Also movement has to be in the same path direction for each end.